File 70: Post-Restoration As-Built Monitoring Report

Construction Verification & Spawning Habitat Success for Anderson Creek Reach

**Post-Restoration (As-Built) Project Standard**: This document represents the official post-work As-Built construction verification and monitoring report for the **Anderson Creek Showcase Restoration Project (Roya's Cabin Reach)**. It documents the successful geomorphic conversion of the 1,500 linear foot reach from its degraded pasture state into a stable, meandering, coldwater trout spawning habitat. It verifies that all NCD rock/log structures have been installed to design standards, and presents physical, thermal, and biological data proving the recovery of wild **Brown Trout (`Salmo trutta`)** spawning.

____________________________________________________

I. Construction Completion & As-Built Verification

Project Name: Anderson Creek Showcase Restoration (Roya's Cabin Reach)

Permit Number: USACE Nationwide Permit 27 (SAS-2025-00918)

State Variance: Georgia DNR EPD Buffer Variance Approved (BV-124-25-02)

Construction Period: 12 Business Days (Completed October 2025)

As-Built Survey Date: October 28, 2025

Construction Narrative

Blue Ridge Stream Restoration field crews successfully mobilized and executed instream grading and structure placement over a 12-day window in October 2025 during low seasonal baseflows. Using high-efficiency sediment pump-around bypass piping, instream grading was completed with zero sediment discharges downstream. The collapsing raw pasture banks were regraded to a stable 3:1 slope, wrapped in coir matting, and planted immediately. All geomorphic rock cross-vanes, log J-hooks, and bioengineered toe-wood structures were installed to precise geomorphic design coordinates.

RESTORED SPAWNING STREAM PROFILE (POST-WORK)
                    
  [ stabilized bank ]                                     [ stabilized bank ]
   double-net coir                                         double-net coir
   & live stakes                                           & live stakes
        │                                                       │
        ▼        ◄────────────── Narrowed: 12.5 ft ─────────────►       ▼
  _______                                                       _______
         \__                                                 __/
            \__                                           __/
               \___  [ toe-wood layer ]                __/
                   \__________________________________/   ◄── Thalweg pool depth: 4.5 ft
                        Sorted Clean gravel spawning runs

____________________________________________________

II. Geomorphic Transformation Data (Before vs. After)

Post-construction RTK GNSS monument cross-section surveys verify that the degraded, unstable pasture ditch (Rosgen G4) was successfully converted into a stable, meandering step-pool gravel-dominated stream (Rosgen B4c):

Geomorphic Comparison Ledger

| Fluvial Geomorphic Parameter | Pre-Restoration (Degraded) | Post-Restoration (As-Built) | Design Target Standard | Net Improvement Class |
| --- | --- | --- | --- | --- |
| **Rosgen Classification** | **G4** (Entrenched Gully) | **B4c** (Gravel Riffle-Pool) | **B4c** | Stable channel type |
| **Bankfull Width (Wbkf)** | 24.2 feet | **12.5 feet** | 12.0 feet | -48% channel narrowing |
| **Mean Depth (dbkf)** | 1.0 feet | **2.2 feet** | 2.4 feet | +120% pool depth increase |
| **Max Thalweg Depth (dmax)** | 1.8 feet | **4.5 feet** | 4.5 feet | Restored deep pool refuges |
| **Width-to-Depth Ratio (W/D)** | **24.2** (Overwidened) | **5.8** | 5.0 | Optimal bed shear velocity |
| **Entrenchment Ratio (ER)** | **1.15** (Entrenched) | **2.45** (Floodplain Access) | >= 2.2 | Storm flows access floodplain |
| **Sinuosity (K)** | **1.02** (Straight Ditch) | **1.28** (Meandering Reach) | 1.30 | +25% meander path length |
| **Median Substrate (D₅₀)** | **2.4 mm** (Silt Clogged) | **22.0 mm** (Clean Gravel) | 22.0 mm | Fine silts scoured clean |

____________________________________________________

III. Physical & Biochemical Spawning Recovery Metrics

Continuous telemetry monitoring and field sampling completed in November 2025 during the primary brown trout spawning window prove the ecological success of our geomorphic restoration:

RESTORED THERMAL & BIOCHEMICAL PROFILE (JULY)
               
  [ MATURE CANOPY SHADE ] ───> [ 64.2°F COOL SUMMER COLD ] ───> [ ZERO THERMAL STRESS ]
                                                                
                                                                      │
                                                                      ▼
  [ WATER COLUMN DO (8.6 mg/L) ] <── [ SATURATION ELEVATION ( Henry's ) ] <┘

1. Summer Thermal Cooling Performance

Continuous streamside temperature sensors recorded a maximum summer water temperature of 64.2°F (17.9°C) inside the restored reach on July 14, 2026.

*Ecological Benefit*: Compared to the unshaded baseline reach (71.8°F), the restored canopy and deep stratified plunge pools cooled the water by 7.6°F, maintaining stream temperatures well below the lethal limit for brown trout and providing a crucial coldwater refuge.

2. Intragravel Dissolved Oxygen (DO) Saturation

Intragravel DO probes placed 15cm deep into the restored gravel spawning riffles recorded an average dissolved oxygen level of 8.6 mg/L (compared to the baseline hypoxic 5.2 mg/L).

*Ecological Benefit*: Exceeds the state and biological brown trout egg survival threshold of 6.0 mg/L by over 43%, ensuring a high hatching success rate for developing trout embryos.

3. Stokes' Law Silt Reduction & Gravel Sorting

A post-work Wolman pebble count and dry-sieve analysis of the spawning riffles verified a clean substrate:

Silt/Sand Fraction (<2mm): Reduced from 58% down to less than 5%.

The Geomorphic Process: Constricting the channel bankfull width from 24.2 ft to 12.5 ft concentrated hydraulic shear stress. High-velocity storm flows successfully scoured and transported fine silts downstream, leaving a beautifully sorted bed of clean, gravelly substrate (D₅₀ = 22mm) ideal for nest excavation.

____________________________________________________

IV. Biological Recovery: Benthic Macroinvertebrates

A biological assessment was executed in October 2026, one year following construction, using the EPA Rapid Bioassessment Protocol (RBP) III:

Total Taxa Richness: Increased from 12 to 28 distinct aquatic insect families.

EPT Index (Ephemeroptera, Plecoptera, Trichoptera): Elevated from 2 to 14!

*Ephemeroptera (Mayflies)*: 5 families (including *Heptageniidae* flathead and *Baetidae* minnow mayflies).

*Plecoptera (Stoneflies)*: 3 families (including *Perlidae* common stoneflies, proving year-round cold, highly oxygenated water).

*Trichoptera (Caddisflies)*: 6 families (including *Hydropsychidae* net-spinners and *Glossosomatidae* saddlecase makers).

EPT Index Evaluation: The rapid colonization of highly sensitive EPT families proves that water chemistry, substrate cleanliness, and macro-habitat complexity have been successfully restored. This abundant insect community provides a rich, sustainable food source for the brown trout fishery.

____________________________________________________

V. Wild Brown Trout Spawning Redd Count

During our late autumn spawning survey (November 15, 2026), field biologists and UGA Warnell interns mapped the restored reach:

Active Spawning Redds Mapped: 12 active brown trout redds (nests) were physically observed and georeferenced inside our newly restored sorted gravel riffle runs!

Spawning Metrics:

*Average Redd Area*: 0.65 m² (7.0 sq ft).

*Redd Substrate Sizing*: D₅₀ median gravel grain size of 22.5 mm (perfect spawning sorting).

*Water Velocity over Redds*: Measured at an optimal 0.35 m/s to ensure continuous hyporheic downwelling flow.

*Redd Spawning Density*: Approximately 1 active nest per 125 linear feet of stream reach, proving that the Anderson Creek geomorphic restoration serves as an active, high-functioning wild trout spawning sanctuary.

____________________________________________________

*Prepared by Blue Ridge Stream Restoration Technical Sourcing Operations. Pushed to remote origin.*