File 71: Creekside Stream Monitoring Field Data Sheet

Field Data Collection Form for Fluvial Geomorphology & Wild Trout Spawning Assessments

**Stream Monitoring Field Form Standard**: This document represents the official **Creekside Stream Monitoring Field Data Sheet (File 71)** for the **Anderson Creek Showcase Restoration Project (Roya's Cabin Reach)**. It provides Hunter Morris, Trout Unlimited, and university interns (UGA Warnell, Georgia Tech) with a ready-to-print, structured field data sheet to systematically record Wolman pebble counts, EPA RBP benthic macroinvertebrates, physical water parameters, and trout spawning nests (redds).

____________________________________________________

I. Project & Station Header Information

Project Name / Reach: Anderson Creek Showcase Restoration (Roya's Cabin Reach)

Station ID (HUC-8): GA-03150105 / Station # ________ (e.g. Monument #01)

Field Observers / Crew: ______________________________________________

Sampling Date: ____ / ____ / 202__   Time: ____ : ____ AM/PM

Weather Conditions: [ ] Sunny  [ ] Partly Cloudy  [ ] Overcast  [ ] Raining

Air Temperature: ______ °F   Recent Precipitation (24 hr): ______ inches

____________________________________________________

II. Hydrological & Biochemical Water Quality Parameters Log

*Equipment Calibrated Prior to Field Work? [ ] Yes  [ ] No  (YSI ProDSS Calibration Complete)*

| Parameters | Unit | Reading #1 | Reading #2 | Reading #3 | Arithmetic Mean | Standard Deviation |
| --- | --- | --- | --- | --- | --- | --- |
| **Water Temperature** | °F (°C) |  |  |  |  |  |
| **Dissolved Oxygen** | mg/L |  |  |  |  |  |
| **Turbidity Index** | NTU |  |  |  |  |  |
| **Specific pH** | s.u. |  |  |  |  |  |
| **Stream Flow Velocity** | ft/s (m/s) |  |  |  |  |  |
| **Gauge Stage Height** | ft (m) |  |  |  |  |  |

____________________________________________________

III. Wolman Pebble Count Tally Grid

*Measure 100 random bed substrate particles across the cross-section riffle profile to compute median diameter (D₅₀)*

| Particle Class | Size Range (mm) | Sieve Classification | Tally (e.g. III, IIII, etc.) | Total Count |
| --- | --- | --- | --- | --- |
| **Silt / Clay** | < 0.062 mm | Silt |  |  |
| **Very Fine Sand** | 0.062 - 0.125 mm | Sand |  |  |
| **Fine Sand** | 0.125 - 0.25 mm | Sand |  |  |
| **Medium Sand** | 0.25 - 0.50 mm | Sand |  |  |
| **Coarse Sand** | 0.50 - 1.0 mm | Sand |  |  |
| **Very Coarse Sand** | 1.0 - 2.0 mm | Sand |  |  |
| **Very Fine Gravel** | 2.0 - 4.0 mm | Pebble (Spawning) |  |  |
| **Fine Gravel** | 4.0 - 5.7 mm | Pebble (Spawning) |  |  |
| **Medium Gravel** | 5.7 - 8.0 mm | Pebble (Spawning) |  |  |
| **Medium Gravel** | 8.0 - 11.3 mm | Pebble (Spawning) |  |  |
| **Coarse Gravel** | 11.3 - 16.0 mm | Pebble (Spawning) |  |  |
| **Coarse Gravel** | 16.0 - 22.6 mm | Pebble (Spawning) |  |  |
| **Very Coarse Gravel** | 22.6 - 32.0 mm | Pebble (Spawning) |  |  |
| **Very Coarse Gravel** | 32.0 - 45.0 mm | Pebble (Spawning) |  |  |
| **Very Coarse Gravel** | 45.0 - 64.0 mm | Pebble (Spawning) |  |  |
| **Small Cobble** | 64 - 90 mm | Cobble |  |  |
| **Medium Cobble** | 90 - 128 mm | Cobble |  |  |
| **Large Cobble** | 128 - 180 mm | Cobble |  |  |
| **Very Large Cobble** | 180 - 256 mm | Cobble |  |  |
| **Small Boulder** | 256 - 512 mm | Boulder |  |  |
| **Medium Boulder** | 512 - 1024 mm | Boulder |  |  |
| **Large Bedrock** | > 1024 mm | Bedrock |  |  |
| **TOTAL COUNT** | — | — | — | **100** |

____________________________________________________

IV. EPA RBP Benthic Macroinvertebrate Tally Card

*Conduct a standard 3-kick net composite sample in gravel riffles. Sort and count sensitive EPT aquatic insects*

1. Group 1: Sensitive Taxa (EPT Index - Indicator of clean, cold trout waters)

Mayflies (Ephemeroptera):

*Heptageniidae* (Flathead Mayfly): ______   *Baetidae* (Minnow Mayfly): ______   *Ephemerellidae* (Spiny Crawler): ______

Stoneflies (Plecoptera):

*Perlidae* (Common Stonefly): ______   *Peltoperlidae* (Roach-like Stonefly): ______   *Nemouridae* (Forest Stonefly): ______

Caddisflies (Trichoptera):

*Hydropsychidae* (Net-spinner): ______   *Glossosomatidae* (Saddlecase): ______   *Limnephilidae* (Northern Case): ______

2. Group 2: Moderately Tolerant Taxa

Dragonflies / Damselflies (Odonata): ______

Crayfish (Decapoda): ______

Dobsonflies / Hellgrammites (Corydalidae): ______

Blackflies (Simuliidae): ______

3. Group 3: Highly Tolerant Taxa (Indicator of siltation and hypoxia)

Midges (Chironomidae): ______

Aquatic Worms (Oligochaeta): ______

Leeches (Hirudinea): ______

4. Biological Health Calculations

Total Taxa Richness: ______ distinct families observed.

EPT Index Score: ______ (Sum of Mayfly + Stonefly + Caddisfly families).

*Evaluation*: EPT >= 12 = Excellent; 8 - 11 = Good; 4 - 7 = Fair; <= 3 = Poor.

____________________________________________________

V. Wild Trout Spawning Nest (Redd) Log Sheet

*Walk the riffle meanders in late autumn. Identify and map clean gravel pockets used by spawning trout*

| Redd # | Geospatial Coordinates (Lat/Lon) | Water Depth (ft) | Flow Velocity over Redd (ft/s) | Redd Length (ft) | Redd Width (ft) | Parent Trout Species (Brown/Brook) |
| --- | --- | --- | --- | --- | --- | --- |
| **01** |  |  |  |  |  |  |
| **02** |  |  |  |  |  |  |
| **03** |  |  |  |  |  |  |
| **04** |  |  |  |  |  |  |
| **05** |  |  |  |  |  |  |
| **06** |  |  |  |  |  |  |
| **07** |  |  |  |  |  |  |
| **08** |  |  |  |  |  |  |

Redd Mapping Field Notes

*(Describe any sediment deposits near redds, presence of spawning parent trout, canopy shading levels, or wood debris structures nearby)*

_____________________________________________________________________________________________________

_____________________________________________________________________________________________________

_____________________________________________________________________________________________________

____________________________________________________

*Prepared by Blue Ridge Stream Restoration Technical Sourcing Operations. Pushed to remote origin.*