File 34: GT Intern Workstream — Page 3: AutoCAD Civil 3D Bioengineering Structures Design

**Hydrologic Engineering & Geomorphic Design Manual**
**Target School**: Georgia Institute of Technology (Georgia Tech) School of Civil & Environmental Engineering
**Project Area**: Upper Savannah Basin & Chattahoochee River Spawning Headwaters
**Supervising Board**: Hunter Morris (Co-Founder & Managing Director) & Hadi Irvani (Board Member)

____________________________________________________

1. Design Philosophy: Natural channel Re-Meandering

In high-gradient trout streams, standard concrete retaining walls or rock rip-rap dumping are ecologically unacceptable. They destroy aquatic habitat, block macroinvertebrate colonization, and reflect high velocities downstream, causing accelerated erosion elsewhere.

Instead, you will draft Natural Channel Design (NCD) and bioengineered structures using native materials: red cedar logs, oak root-wads, and high-strength geogrid soil lifts. These structures stabilize eroding banks, scour deep cold pools, and mimic historic mountain stream features. Your role as a GT engineering intern is to generate precise, AutoCAD Civil 3D plan-sheets, cross-sections, and detail diagrams.

____________________________________________________

2. Technical Drawings & Dimensions for NCD Structures

You will draft four primary bioengineered structures on our site plans:

A. Rock J-Hook Vane (Flow Deflection & Pool Scour)

Description: A rock structure built along the outer bank of a curve. The "vane" portion slopes upstream, directing fast currents away from the bank, while the downstream "hook" scours a deep central pool.

AutoCAD Parameters:

Vane Angle: Set between 20\text{ and } 30\text{ degrees} relative to the upstream bank tangent.

Slope: The rock arm must slope down from the bankfull elevation at the bank to the stream bed at 1/3 the width of the channel.

Hook Rocks: Position the J-hook rocks in the center of the channel, submerged below the baseflow water surface. Leave gaps of 10\text{--}15\text{ cm} between rocks to allow trout passage and scour scour halos.

B. Double Log Cross-Vane (Grade Control & Pool Scour)

Description: A two-arm log structure crossing the channel, sloping upstream to create a central plunge pool and prevent bed incision (erosion of the stream bed).

AutoCAD Parameters:

Log Material: Select fresh red cedar or white oak logs (30\text{--}45\text{ cm} diameter).

Throat Width: The central "weir" gap must span exactly 1/3 the bankfull width of the channel.

Log Arm Slope: Slope the log arms upward at 2\text{--}7\% from the stream bed to the bankfull bank line.

C. Cedar Toe-Wood (Bank Stabilization & Woody Habitat)

Description: Layers of overlapping cedar logs and root-wads placed at the toe of a regraded bank curve, covered with soil lifts.

AutoCAD Parameters:

Placement: Place the cedar logs below the baseflow water line to prevent decay from air exposure.

Root-wad Projection: Extend the fibrous cedar root-wads outward into the stream channel at a 45\text{-degree} angle facing upstream to trap fine gravels and provide cover for large brown trout.

____________________________________________________

3. Structural Anchoring & Geotech Calculations

To ensure these cedar structures do not wash away during major floods, you must execute the Log Gravity Anchoring Calculations.

LOG GRAVITY ANCHOR PROFILE
                       
                      BANKFULL WATER STAGE ~~~~~~~~~~~
                                       |
                   [CEDAR LOG]         | Water Depth (H)
                    Volume (V)         |
                                       v
        ============== BEDROCK / CHANNEL BED ==============
                       |               |
         Threaded      |               | Expansion Wedge Bolt
         Epoxy Anchor  |=======#=======| (High-Strength Grout)
                               |
                               | Heavy Steel Cable
                               v

The buoyant upward force (F_b) acting on a submerged log must be completely countered by the downward gravitational force of the soil/rock overburden and mechanical anchor anchors.

Anchor Safety Factor Equation:

FS_{anchor} = \frac{W_{overburden} + W_{log\_dry}}{F_{buoyant}} \ge 1.5

Where:

F_{buoyant} = \gamma_{water} \cdot V_{log}, where V_{log} = \pi \cdot R^2 \cdot L and \gamma_{water} = 62.4\text{ lb/ft}^3.

W_{log\_dry} = \gamma_{cedar} \cdot V_{log}, where dry red cedar weight \gamma_{cedar} \approx 23.0\text{ lb/ft}^3.

W_{overburden} = \text{weight of rocks/soil placed on top of the log keys in the stream bank}.

Worked Example:

Calculate the buoyant force and required anchoring weight for a cedar log of radius R = 0.75\text{ feet} and length L = 12.0\text{ feet}:

#### 1. Calculate Log Volume (V_{log}):

V_{log} = \pi \cdot (0.75)^2 \cdot 12 = 21.21\text{ ft}^3

#### 2. Calculate Buoyant Force (F_{buoyant}):

F_{buoyant} = 62.4 \cdot 21.21 = 1,323.5\text{ lbs of upward force}

#### 3. Calculate Log Dry Weight (W_{log\_dry}):

W_{log\_dry} = 23.0 \cdot 21.21 = 487.8\text{ lbs of downward dry weight}

#### 4. Calculate Required Overburden/Mechanical Anchor Weight (W_{req}) for FS = 1.5:

1.5 = \frac{W_{req} + 487.8}{1,323.5} \implies W_{req} = (1.5 \cdot 1,323.5) - 487.8 = 1,497.45\text{ lbs}

Your design must specify a minimum of 1,500 lbs of mechanical cable-anchors (epoxied into bedrock or secured using concrete/rock deadman anchors in the bank) to satisfy the FS_{anchor} \ge 1.5 safety margin.

____________________________________________________

4. Geotechnically-Reinforced Soil Lift (GRSL) Details

To rebuild vertical, stable banks along outer bends, you will detail Geotechnically-Reinforced Soil Lifts (GRSL):

Core Fabric: Specify high-durability, double-walled coir fiber geotextile wrap (Type 700 or 900).

Filling: Pack the inside of the fabric wraps with bank-run compacted gravel (15\text{-cm} base) and rich organic soil (15\text{-cm} top) to promote native root growth.

Compaction: Specify 95\% Standard Proctor Density compaction for the soil core.

Interlocking: Wrap the coir fabric around the soil layers in overlapping envelopes, stepping the lifts back at a 1:0.5 (Horizontal to Vertical) slope. Interleave native willow stakes and tag alder branches between the lifts to bind the system mechanically.